This checklist identifies the criteria to be satisfied before standing tubing with a worker in the derrick or on the tubular racking board during snubbing operations. Refer to IRP15 Section 15.8.11 for more information.

Well Operations

- No Hydrocarbons present at surface

- No workers on the tubing board when the tubing is pipe light

- No workers on the tubing board when stripping rams are required to stage tubing in/out of the well

Primary Blow Out Preventers

- All primary BOP’s have been pressure tested to jurisdictional regulations

- Primary pipe rams have a functional, tested and documented ram saver

- BOP configuration meets requirements of IRP 15 Snubbing Operations

Egress System

- Tubing board has a rear egress system in place that does not require the worker to disconnect from primary fall arrest equipment used during tripping operations

- Snubbing basket has egress systems in place that meets the criteria outlined in IRP15 section 15.8.3 Emergency Egress Systems

Ram Saver

- Audible alarm indicating secondary ram position change (open/close)

- Visual indicator on secondary ram position

- Throttle interrupt that cuts the rig throttle when secondary rams begin to close

- Ram Saver system is functional, tested and documented

Slip Interlock

- In place on all snubbing slips and heavy slips

- Slip Interlock in use, functional, tested and documented

Tubing Isolation

- Plug installed meets requirements found in IRP 15 with dual barrier

- Plug and seal integrity has been confirmed by a documented/recorded pressure test for 10 minutes

Tubing Integrity

- Number of joints to be snubbed is calculated and communicated to all workers

- Maximum stroke length to be used while snubbing has been calculated

- BOP and slip configuration has been confirmed to be acceptable for calculated buckling force exerted on the tubing string while snubbing

- Tubing type and grade has been confirmed acceptable for maximum well pressure expected, including surface pressure for buckling calculations and bottom hole pressure for resistance to buckling/collapse

| Working pressure (wellhead) |  |  | Number of joints to be snubbed |  |
| --- | --- | --- | --- | --- |
| Plug Type Used |  |  | Tubing size and grade |  |

| Service Rig Company & No. |  | Rig Manager (print) |  | Rig Manager (sign) |  | Date |
| --- | --- | --- | --- | --- | --- | --- |

| Snubbing Service Provider & No. |  | Snubbing Supervisor (print) |  | Snubbing Supervisor (sign) |  | Date |
| --- | --- | --- | --- | --- | --- | --- |

| Prime Contractor & Location |  | Wellsite Supervisor (print) |  | Wellsite Manager (sign) |  | Date |
| --- | --- | --- | --- | --- | --- | --- |

IRPs are available from Energy Safety Canada at www.energysafetycanada.com